ERRATA FOR GEOCHEM-PC, VERSION 2.0

To date, several errors in v. 2.0 of GEOCHEM-PC have been called to our attention, and these are briefly described below.  Because most of these can be circumvented (or ignored) by individual users, we are not issuing a new release of the program or database at this time.  We continue to solicit your comments regarding any problems with the program; these will be added to our current list and appropriate changes made in a future program version.

Program Errors

1.	The procedure for imposing/disallowing simple solids does not work as described in our usage notes, but only when more than one solid is to be specified (entry in line 8 is greater than 1).  For these cases, each solid to be imposed/disallowed must be entered on a separate line following the descriptive comment for LINE 27.  Thus, if three solid phases are to be disallowed, there should be three lines of data (one per solid) in the position occupied by line 27, as per the following example:

.

.

LINE 27. Imposed/disallowed solid phases: see manual (neg. value = disallow)

1 1 -1

1 9 -3

6 9 -1

LINE 28. Number of redox reactions to consider; used only if item 10 = 'y'

.

.

2.	Redox reaction number 19 [S2- = S0(s) + 2e-] will not be computed correctly by the program.  To the best of our knowledge, this is the only redox reaction included with v. 2.0 that does not work properly.  Any similar reactions (e.g., oxidation of selenide to elemental Se) should likewise be avoided.  A solution to this problem will be included as part of the next program version.

Database Errors (files GEODAT-A.LIG and GEODAT-A.MET)

Please note the following four corrections to the thermodynamic database files included with version 2.0.

1.	The ionic charge of oxalate (ligand 18) should be specified as 2- in the list of ligands at the beginning of the database.  Columns 4-6 should thus be edited to read '-02'.

2.	The ionic charge of Ferrozine (ligand 78) should be specified as 2- in the list of ligands at the beginning of the database.  Columns 4-6 should thus be edited to read '-02'.

3.	The log K for Al3+ + 2SO42- = Al(SO4)2- should be 5.0, not 5.4, to be consistent with Nordstrom and May (1989).

4.	The log K for alunite (mixed solid number 18) should be 1.4, not 1.6.

D. R. Parker

July 3, 1997

USAGE NOTES FOR GEOCHEM-PC, VERSION 2.0

INTRODUCTION

GEOCHEM-PC is a descendant of the multi-purpose chemical speciation program GEOCHEM, which has been widely-used by soil and environmental chemists.  The PC-compatible version of GEOCHEM has evolved through several versions which have been passed from user to user on an informal basis, and has become a functional and increasingly convenient research tool that we have distributed to interested users in the U.S. and abroad.  These usage notes are a preliminary document to permit usage of the latest version of the program.  We still hope to develop a more comprehensive user's manual that will provide complete documentation, but the likely completion date of that document is unclear at this time.  A chapter in an upcoming Soil Science Society of America Special Publication is presently the best reference for citation of the program.

A number of changes and improvements have been made since v. 1.23 (and versions 1.24a, 1.25, 1.26, 1.27, and 1.5 which enjoyed limited distribution) of GEOCHEM-PC.  These notes are intended to highlight the new features, and are aimed primarily at experienced GEOCHEM and GEOCHEM-PC users, to whom most of the program's structure and conventions will look familiar.  The section  'Program Files and Usage' should be read carefully for clarification of the changes that have been made.

Until a more comprehensive manual is developed, new users may be able to employ GEOCHEM-PC by carefully reading all of this document, and by initially running the program in the interactive, '20-questions' mode.

The program and accessory files are designed to be used as a package.  Compatibility problems with older versions of GEOCHEM-PC are documented in the following section, but use caution when attempting to utilize old input data files or thermodynamic databases.  To ensure success, use only the versions of these files that accompany version 2.0 of the program.

Our use of GEOCHEM-PC, as well as the alterations we have made, largely reflect our research interests in plant mineral nutrition, phytotoxic trace elements, soil solution chemistry, and related topics.  We have created a program for comparatively painless use by soil and plant scientists who do not have extensive experience in the theory and use of computerized speciation models.  Program features and the component species included in the program database are oriented towards such use, as will be any future developments and improvements.  We welcome any comments and criticisms of this version of GEOCHEM-PC.

PROGRAM FILES AND USAGE

Installation and Setup

The executable program for GEOCHEM-PC, Version 2.0, has been compiled using MicroSoft Fortran (v. 5.0), and should run successfully on any IBM or compatible PC operating under DOS 3.0 or higher.  At least 512 kB of main (board) memory is recommended.  The executable version does not require a math coprocessor, but will utilize one if it is present.  On computers without a coprocessor, an emulator is invoked for floating-point math, but the program is rather slow.  A hard disk is highly recommended, but the program can be run using one or two floppy-disk drives.

The thermodynamic database is an ASCII file that presently occupies about 140 kB of disk space.  It can be edited with any ASCII editor or ASCII-compatible word processor capable of handling a file of this size with a 132-character line length.  The configuration file (GEOCHEM.CFG) and the example input file (MASTEXPL.DAT) are small, 80-column ASCII files.

The distribution diskette should contain the following files and subdirectories:

GEORUN.EXE	The executable version of GEOCHEM-PC.

GEOCHEM.CFG	A small ASCII file for configuring the program.

GEODAT-A.MET	Two copies of the thermodynamic database, one 	GEODAT-A.LIG	sorted by metal, one by ligand (see the Thermodynamic Database section below).

MASTEXPL.DAT	A 'master' example file for data input.

EXAMPLES    <DIR>	A collection of 8 example data input files that illustrate the program's major features.  See the ASCII text file README.EXP for a description.

PRNTUTIL    <DIR>	Several utilities for printing the program output in a condensed font to accommodate the 132-character line length.  See the ASCII text file README.PRT for documentation.

For hard disk use, we recommend copying the entire disk, along with the accessory subdirectories if desired, to a dedicated subdirectory such as C:\GEOCHEM (the DOS command XCOPY is a convenient way to do this).  For floppy disk use, we strongly urge users to make a working copy of the diskette, and retain the distribution diskette as a backup.

Be sure to edit the GEOCHEM.CFG file to conform to your setup before attempting to run the program.

The GEOCHEM.CFG File

An auxiliary file for initial configuration of GEOCHEM-PC, called GEOCHEM.CFG, must be located in the same directory or subdirectory as the main program, GEORUN.EXE.  This ASCII file is read by the program to obtain the drive/path specifications for the various files required by GEOCHEM-PC.  With this feature, input and output files can be located on any drive (hard disk, floppy disk, RAM drive, etc.), and in any subdirectory.

GEOCHEM.CFG can be readily edited with any ASCII editor.  The file itself should be fairly self-explanatory; comment lines are preceded by a single quotation mark.  There are four drive/paths to be specified, as well as  a default name for the thermodynamic database file (although this can be altered during the program run).  Please note that the maximum length of the drive/path strings read by GEOCHEM-PC is 30 characters.  This permits specification of a four-level subdirectory (root directory plus three 'branches') if 8-character directory names are used.

Data Input

GEOCHEM-PC can be run in either of two modes for data input: (1) a strictly interactive mode wherein all input data are entered in response to questions posed by the program, or (2) a data file input mode.

For either mode, GEOCHEM-PC is started by typing 'GEORUN'.  The program will ask whether you desire interactive (option 'S' for screen) or file (option 'F') input, and prompt you for the file name in the latter case.

Please note:  Our experience with version 2.0 is that good, accurate guesses of the free metal and ligand concentration for the first case are essential to successful computation.  This is particularly crucial with systems containing chelating agents such as EDTA which can lower the free concentrations of metals such as Fe(III) and Cu(II) to values below 10-15 mol L-1.

Interactive Mode

In this mode, the questions posed by GEOCHEM-PC are quite explicit, and give clear instructions as to how the various input items should be formatted (e.g., the need for negative logarithms of concentrations).  For first-time users, the interactive mode forces stepwise consideration of the required input that should make use of the data file option easier.

Data File Mode

For large data sets, for flexibility in data entry, for repetitive calculations, and for ease of editing, most users will find the use of ASCII input data files more convenient.  This feature is especially valuable for problems of a hypothetical or theoretical nature that require repetitive runs with changes in only selected values, such as pH, pE, or pCO2.  In addition, errors in the input can be corrected easily so that problems can be rerun without burdensome re-typing of all data.

The input data file is an ASCII text file named xxxxxxxx.DAT, where xxxxxxxx is any 8-character string of the user's choice.  As a default, the output file is assigned the same name with the extension .OUT, but the user may override this and enter another name for the output file.   The file MASTEXPL.DAT (MASTer EXamPLe file, see Appendix A) illustrates the required format, as well as how the various options are specified during input.  Each data line is preceded by a numbered descriptive comment to assist the user in editing the input file.  Note that lines numbered 1 through 22 are always read by the program, and should be appropriately edited for every program run.  Lines 23 through 31 are ignored by the program unless the options selected previously require their use.  They need not be edited for every run, but should always be retained in their proper order in the file to ensure that the program can correctly locate all required data.  We recommend using MASTEXPL.DAT as a template for all input data files by copying it to a new .DAT file prior to editing.

Please note that the MASTEXPL.DAT file is not intended to be used in an actual program run as Gibbs phase rule violations will occur.  It is only included as a template from which other, usable files can be created.

Also note that input data files from most previous versions (1.27 and earlier) will not work with version 2.0, unless they are modified as follows:

1.	Insert the four lines (2 descriptive, 2 actual data lines) at the beginning of the file that provide the  two lines of user-specified documentation to appear in the .OUT file for each program run.  See MASTEXPL.DAT.

2.	Delete the four lines (2 descriptive, 2 actual data lines) pertaining to adsorption of metals and ligands on solid surfaces.  We have deleted this rather limited feature from the program.

3.	Change the selection for output of complex concentrations from alphabetical (y or n) to numerical (0, 1, 2, or 3).  See MASTEXPL.DAT for an explanation of the numeric values.

Interactive Alteration of log K Values

GEOCHEM-PC provides users with an opportunity to interactively alter stability constants for complexes and simple solids during a program run.  Sensitivity analyses of the effect of variation in the stability constants on computed speciation is thus more convenient because repeated editing of the thermodynamic database is not required.  The questions regarding alteration of any log K values are posed and answered interactively.  Whenever you run the program, it will query you about this option just before it reads in the thermodynamic database file.

Program Output

The output from a single program run consists of one ASCII file containing a number of tables; the line length is 132 characters.  GEOCHEM-PC always produces two tables every time the program is run.  The first summarizes the program run, including all of the input concentrations and master variables (e.g., pH, pE), as well as the program version number, current date and time (from computer's clock), and the input, output, and database file names in use.  The second lists all of the thermodynamic stability constants used, in a format similar to that of the database.  Note that this is always the table of infinite-dilution (activity-based) constants, and lines no longer appear for metal-ligand  combinations for which there are no solids or complexes.

Four other tables for each individual case can be generated, if specified.  These are assigned numbers such as x-A, x-B, xC.., where the Arabic number indicates the case number.  They are:

x-A	The 'case progress' table that provides the free activity, free concentration, and total concentration of each component species, along with pH and other data such as the amounts of precipitated solids and the computed charge balance of the solution.

x-B	Tables of concentrations and/or activities of soluble complexes in both negative common logarithm and exponential format.

x-C	The 'overall speciation' table is actually a pair of tables giving the concentrations of free metals and ligands, and metal-ligand complexes.  They differ in that the first expresses the concentration of complexes as the total concentration of each metal complexed with a given ligand, while the second gives the total concentration of each ligand complexed by a given metal.  This dichotomy is necessary to accommodate the stoichiometry of complexes with multiple metal or ligand moieties, such as M2L or ML3.  For metal-ligand pairs that only form 1:1 complexes, the values in the two tables are identical.

x-D	The 'primary distribution' table that summarizes the distribution of each metal and ligand among its major species, expressed as a percentage of the total metal or ligand present.  For simplicity, only values of 0.01% or greater are listed.

A table of concentration constants (corrected for ionic strength) can be optionally selected; it will appear once for runs using fixed ionic strength, and as Table x-E for each individual case when I is computed by GEOCHEM.  Tables of interaction capacities and/or intensities (see Stumm and Morgan [1981] or Sposito and Mattigod [1980]) can also be selected, and appear as Table x-F for each case.

The Thermodynamic Database

A number of significant changes have been made to the thermodynamic database file (generically referred to as GEODATA) for GEOCHEM-PC.  In part, we wished to make the database easier to read and to edit without error, and have thus expanded this ASCII file from 80 to 132 column format.  The two versions of the file included with v. 2.0, GEODAT-A.MET and GEODAT-A.LIG, contain identical stability constants, but are organized somewhat differently as discussed below under 'Editing'.  We recommend that users select one version for use, and discard the other.  Please note that most older versions of the GEODATA file will not work with version 2.0.

Use

For general use, the user can now specify the name of the database file, such that different versions of the database can be maintained and used concurrently.  As noted above, the GEOCHEM.CFG file should contain a default name for the database file most commonly used.  Please enter the full file name (including extension) in the GEOCHEM.CFG file, or when asked to do so by the program.

A list of metals and ligands currently included in the database is given in Appendix B.  Tables of the redox and mixed solid reactions presently included can be found in the 'Program Features and Options' section below.

Editing

For users who wish to check and/or edit values in the database, the following summarizes the major features of the file.  The database is an ASCII file that can be edited with any ASCII editor or ASCII-compatible word processor capable of handling a file of this size with a 132-character line length.  New users are urged to proceed cautiously when editing, and to back up the database file frequently.  Corruption of the file by embedding tabs or control characters, or by 'wrapping' of the 132 column lines, will cause program failure.

The file contains comment lines at the beginning of each section of the database, and these provide space for user notes and internal documentation of formats and database restrictions.  Please note that the number of these comment lines appearing before each section should not differ from those in the database files included with v. 2.0.  Also note that all stability constants should now be entered as the actual values of log K with explicit decimal point (rather than as integer values equal to 10 log K or 100 log K as in prior versions).

The four primary sections of the database file in the order that they appear are:

1.	The list of all metals, and then all ligands, contained in the database.  It is no longer necessary to keep count of the number of metals and ligands; GEOCHEM-PC recognizes ligand 99 (OH-) as the last entry.  The 6-character names (which are reproduced in the output tables) are now specified here, rather than as part of the program source code.  Users may thus add, delete, or substitute metals or ligands, up to a maximum of 50 metals and 99 ligands, and may edit the 6-character code names to suit their preferences.  The format of these lines should always be as follows: the metal or ligand code number (columns 1-2), the ionic charge (columns 4-6), the ionic radius in F6.2 format (not presently used by the program) (columns 8-13), and the metal or ligand name (columns 15-20).  Columns 25-132 provide additional space for formal or IUPAC names; these are for internal documentation and convenience in managing the database, and do not appear in the program output.

2.	Following two lines that provide the log K's for the hydrolysis of water and for CO32--CO2(g) equilibrium, the main section of the database provides the log K values for simple solids (up to 3) and soluble complexes (up to 6).  The log K values may now be entered with a precision of two decimal places, when justified.  The format expected by GEOCHEM-PC is F6.2; values up to 999.99 and down to -99.99 can thus be entered.  After each value, the stoichiometric coefficients for the metal, ligand and H+ or OH- (format I1,I1,I2) should appear.  Columns 97 to 132 are reserved for annotations regarding the sources of the constants.  The two versions of the database, GEODAT-A.MET and GEODAT-A.LIG, differ only in this section.  In the former the lines are sorted by increasing metal code number, such that all the data for Ca appears first, followed by that for Mg.  In the latter the sorting is by ligand code number, such that all data for CO3-2 appears first.

Please note:  A 'dummy' line for metal 50 (H+) and ligand 99 (OH-) must appear as the last entry of this section.  This is how the program recognizes the end of the section, and precludes the need for the user to keep count of the total number of entries, thus eliminating a frequent source of error encountered with earlier versions of the database.

3.	The third section is for redox constants and has been modified only slightly from the original version of GEOCHEM.  The user is referred to the original GEOCHEM manual for clarification.

4.	Similarly, the fourth section, for mixed solids, resembles the original version, and the old manual should be consulted.

We felt it was crucial to provide attributions for all stability constants that we have altered, added, or checked.  Our method of indicating and referencing these attributions includes (i) brief  annotations at the end of each line of data, (ii) footnotes (referenced by metal and ligand code numbers) when more explanation is required, (iii) a list of literature references including a short-hand code for each reference, and (iv) a list of the people (coded by their initials) who have contributed to the selection of data.  An example of the first item appearing at the end of a line for a metal-ligand pair is:

WAN:Li79(solids); DRP:St81(cmplxs)

indicating that one of us (WAN) has selected log K values for all simple solids from Lindsay (1979), while another (DRP) has chosen values for all of the complexes from Stumm and Morgan (1981).  If no end-of-line annotation is present, the log K values on that line have not yet been checked or evaluated by us.  We have relied mainly on published compilations as sources of thermodynamic data such that, in general, our selection of log K values should not be viewed as a critical assessment of the original literature.  Rather, we have sought to correctly identify the immediate sources of the data included.  The section of footnotes further explains certain selections, and identifies solid phases when their identity is not readily apparent.  We have taken pains to ensure that our values represent log K's at zero ionic strength as required by the program.  We encourage users to notify us of apparent errors or improved estimates for any of the values in the database.

Other Changes

A coding error prevented GEOCHEM-PC from switching to the Helgeson equation for activity coefficient computations at ionic strengths greater than 0.5 mol L-1.  This was corrected in version 1.26.

PROGRAM FEATURES AND OPTIONS

GEOCHEM-PC is designed to perform equilibrium speciation computations for soil solutions, hydroponic solutions for the rearing of plants, natural waters, and similar aqueous systems.  Like its predecessors, GEOCHEM-PC is intended for equilibrium calculations at constant temperature and total pressure, generally 298 K and 0.1 MPa, respectively.  The program cannot adjust the thermodynamic data for changes in T and P, although alternative versions of the database could be created and used for systems at other constant temperatures and pressures.  The 'case' structure (i.e., 10 different cases per program run) is particularly convenient for examining the effects of master variables such pH, pCO2, and pE on the speciation of a given solution.

GEOCHEM-PC is computationally similar to GEOCHEM and REDEQL, and these programs have been adequately described elsewhere (Morel and Morgan, 1972; Sposito and Mattigod, 1980; Sposito, 1984).  Briefly, a requirement for mole balance for all component species,  coupled with conditional (ionic-strength adjusted) stability constants for the complexes and solids which may form from these components, provide a set of nonlinear equations. These equations are solved simultaneously using a variation of the Newton-Raphson method, until the sum of the concentrations of all metal and ligand species agree with their input concentrations to within a degree of precision selected by the user (the convergence criterion, usually 0.01%).

The program does not impose or require electroneutrality, nor its close relative, the proton condition (Stumm and Morgan, 1981).  This characteristic makes the program 'forgiving' when using input data which are incomplete or exhibit poor charge balance (Sposito, 1984), but prevents direct solution of certain types of problems.  We have improved upon this compromise by including a subroutine that computes the net charge balance after convergence by summing all positive and negative species.  This provides several benefits, including an enhanced ability to determine the speciation in some types of solutions which freely exchange CO2 with the atmosphere (see below).

In GEOCHEM-PC, the infinite-dilution, activity-based equilibrium constants from the thermodynamic database are converted to concentration constants with single-species activity coefficients for each reactant and product species.  When the ionic strength is  0.5 mol L-1, the Davies (1962) equation is used to estimate activity coefficients of charged species, while a modification of the Helgeson (1969) equation is invoked for systems in which I > 0.5 mol L-1 (Sposito and Mattigod, 1980).  Activity coefficients for neutral species are approximated according to Harned and Owen, (1958, p. 531-532), with km fixed at 0.1 (Sposito, 1984).  When ionic strength is computed from the input data, the conditional equilibrium constants are recomputed after each program iteration if the current value of I differs from that of the previous iteration by more than 0.5%.  Thus, convergence is not reached until the computed speciation, the ionic strength, and the conditional constants are stable and self-consistent.

Soluble Complexes and Simple Solids

During a single program run, GEOCHEM-PC can simultaneously consider up to 20 metals and 30 ligands.  For each metal-ligand pair, up to six soluble complexes and up to three different simple solid phases (complexes and solids which contain no more than one component metal, one component ligand, and H+ or OH- ions) can be considered.  The thermodynamic database allows for up to 50 individual metals and 99 different ligands.

The conventions followed by GEOCHEM-PC are best gleaned from the example reactions in Table 1.  Each complex or simple solid formed by a metal and a ligand is designated by three stoichiometric coefficients (e.g., 13 0 or 11-1); the first is the number of metal ions, the second is the number of ligands, and the third is the number of protons consumed (if positive) or released (if negative) in the formation reaction.  Inspection of Table 1 reveals that GEOCHEM-PC 'thinks' in terms of formation reactions, i.e. association reactions for complexes and precipitation reactions for solids.  All acid-base reactions are handled via proton exchange, and reactions that could be written as consuming OH- must be written instead as hydrolysis reactions releasing H+.  All reactions must be expressed using the  log K values for overall, not stepwise, reactions.  Lastly, note that the program is indifferent to characteristics of solid phases other than reaction stoichiometry and solubility (see, e.g., the three Fe oxides/hydroxides given at the end of Table 1).

The input data file EXAMPLE1.DAT included on the distribution diskette provides an example where both complex and simple solid phase formation are considered.

Table 1.  Examples of fundamental complexation and precipitation reactions considered by GEOCHEM-PC, illustrating the conventions used.

Stoichiometric

Coefficients	log K

I.  ACIDS AND BASES

a.  Weak acid

CH3COO- + H+  =  CH3COOH (acetic acid)	01 1	4.8

b.  Weak base

NH3 (aq) + H+  = NH4+	01 1	9.3

II.  COMPLEXATION REACTIONS

a.  Simple complex

Ca2+ + SO42-  =  CaSO40	11 0	2.3

b.  Mixed complex -- MLH type

Ca2+ + PO43- + 2H+  =  CaH2PO4+	11 2	21.0

c.  Mixed complex -- MLOH type

Fe3+ + DTPA5- + H2O  =  Fe(OH)DTPA3- + H+	11-1	20.9

III.  METAL HYDROLYSIS

Al3+ + H2O  =  AlOH2+ + H+	10-1	-5.0

Al3+ + 2H2O  =  Al(OH)2+ + 2H+	10-2	-10.1

IV.  SIMPLE SOLID PHASES (one metal and one ligand only, except for H/OH)

a.  Simple solid phase

Ca2+ + SO42- + 2H20  =  CaSO42H2O (gypsum)	11 0	4.6

b.  Simple solid -- MLH type

Ca2+ + PO43- + H+ + 2H20  =  CaHPO42H20 (brushite)	11 1	18.9

c.  Simple solid -- MLOH type

4Mg2+ + 3CO32- +5H2O  =  Mg4(CO3)3(OH)23H2O + 2H+ 	43-2	1.5

(hydromagnesite)

V.  OXIDE AND HYDROXIDE MINERAL PHASES

a.  Metal-hydroxide solid phase

Fe3+ + 3H2O  =  Fe(OH)3 (amorphous) + 3H+	10-3	-3.5

b.  Metal-oxyhydroxide solid phase

Fe3+ + 2H2O  =  α-FeOOH (goethite) + 3H+	10-3	0.02

c.  Metal-oxide solid phase

Fe3+ + 3H2O  =  1/2 α-Fe2O3 (hematite) + 3H+	10-3	-0.09

Imposed/Disallowed Solids

GEOCHEM-PC allows the user to impose or disallow specific simple solid phases. Imposition of a solid, however, merely 'suggests' a likely solid phase to the program at the start of the run.  It is not possible to impose equilibrium with an infinite quantity of a solid phase on the system.  To ensure that a given solid precipitates, excessive concentrations of the appropriate metal and ligand should be used during input. It is possible to specifically impose or disallow only one solid for a given metal-ligand pair; preceding the solid number with a negative sign tells the program that the specified solid is to be disallowed.  Moreover, disallowing one of the three possible solids works with certain limitations.  If solid no. 1 is disallowed, GEOCHEM-PC will ignore solids no. 2 and 3 as well, as if all three had been disallowed.  If solid no. 2 is disallowed, solid no. 1 is considered, but solid no. 3 is ignored.  If solid no. 3 is disallowed, solids 1 and 2 will be considered.  This characteristic can sometimes be utilized advantageously by arranging the three possible solids in decreasing order of solubility in the database (solid 1 is most soluble, 3 is least soluble).  Thus, by disallowing solid 2 (and thus 3), one can 'force' the program to consider the more soluble solid phase (number 1) if it is believed to be the more likely phase based on kinetic or other considerations.  Similarly, disallowing solid 3 permits equilibrium to be established with solid 2.  This feature can not be utilized blindly, however, because the order of solubility of some solid phases (e.g., Ca-phosphates of differing stoichiometry) varies with solution composition.

The input data file EXAMPLE2.DAT included on the distribution diskette provides an example where a simple solid phase has been disallowed.

Computed pH

Although solution pH is routinely measured experimentally, GEOCHEM-PC will compute a predicted solution pH based on the total concentrations of metals, ligands, and a parameter termed 'TOTH' (for total proton concentration).  This feature can be useful for hypothetical cases, such as predicting the final pH after two different waters are mixed.  The TOTH parameter is essentially a restatement of the proton condition:

TOTH (mol L-1)  =    n[acid] + [H+]  -    m[base] - [OH-]

where [acid] is the concentration any species containing n dissociable protons, [base] is that of any species containing m dissociable hydroxyls, [H+] is the concentration of protons from strong acids (such as HCl), and [OH-] the concentration of hydroxyls from strong bases (such as NaOH).  The TOTH parameter should be specified as either positive or negative, to indicate a net 'addition' of either acid or base, respectively.

The TOTH concept can be difficult to grasp initially, but consideration of the examples in Table 2 may be helpful.  The key is that contributions to TOTH are always computed relative to the component species.  Thus, although addition of NaHCO3 is normally viewed as a base (or alkalinity) addition, it actually constitutes an addition of H+ relative to the component species, CO32-.  Similarly, an addition of Na2CO3 does not change the proton condition with respect to CO32-, while increasing pCO2 (and thus H2CO3*) increases TOTH by two protons per dissolved CO2.  Note that the component species for hydrolyzable metals such as Al(III) are generally specified as the completely dissociated ion; hence they do not contribute to TOTH.  Vanadate was included in Table 2 to illustrate that the component species present in the database need not be fully dissociated.  The final pKa for vanadate is so high that the VO43- species would seldom be of interest, so HVO42- was selected as the component species.  Thus, the contributions of Na3VO4 and NaH2VO4 to TOTH are -1 and +1 mol per mol of V, respectively.

The input data file EXAMPLE3.DAT included on the distribution diskette provides an example where the solution pH has been computed for a simple Al salt solution to which either acid or base as been added.

Table 2.  Examples of how contributions to the TOTH parameter should be computed when solution pH is to be computed by GEOCHEM-PC.

Component	Compound

Species	Added	Change in TOTH †

Al3+	AlCl3	0

Al(OH)3	-3

CO32-	Na2CO3	0

NaHCO3	+1

CO2 (H2CO3*)	+2

NH30	NH3 (aq) or NH4OH	0

NH4Cl	+1

CH3COO- (acetate)	NaOAC	0

HOAc	+1

HVO42-	Na3VO4	-1

NaH2VO4 ‡ 	+1

†	The coefficient given multiplied by the molarity of the added compound yields the net change in TOTH in mol L-1.

‡	The salt formed when commercial Na-metavanadate (NaVO3) is dissolved in water.

Ionic Strength

GEOCHEM-PC can be run with either computed or fixed ionic strength.  The latter option is convenient for complex systems where all of the component species may not be known or experimentally determined (e.g., soil solutions).  In such situations, the solution electrical conductivity can be measured and used to estimate I based on empirical relationships; the estimate is then input to the program as the fixed ionic strength.

Open-System Calculations for CO2

In solutions containing inorganic carbonate that are open to the atmosphere, the solution pH, the pCO2, and the concentrations of carbonate species are linked by the following reactions.

CO32- + 2H+  =  CO2 (g) + H2O     	[1]

CO32- + 2H+  =  H2CO3* 	[2]

CO32- + H+  =  HCO3-  	[3]

If any two of the parameters pH, pCO2, and a carbonate species are specified, the others are fixed by, and may computable from, the above equilibria.  In the analysis of water samples, the concentrations of individual carbonate species are not determined directly, but rather are determined from measures such as total inorganic carbon (CT) or total inorganic alkalinity.  The latter is defined as

[Alk] = [HCO3-] + 2[CO32-] + [OH-] - [H+] 	[4]

where, for the moment, the contributions of carbonate ion pairs and complexes have been omitted.

For ease of computation, GEOCHEM-PC treats exchange of gaseous CO2 as the solubility equilibria of a fictitious solid phase, H2CO3, whose solubility is proportional to the partial pressure of CO2.  When H2CO3 'precipitates' during a program run, the program has predicted that dissolved CO2 has left the solution, i.e., escaped to the atmosphere.  When H2CO3 'dissolves', CO2 has entered the solution, i.e., been absorbed from the atmosphere.  Unfortunately, the amount of inorganic carbon in the atmosphere cannot be controlled independently, but only exists as the excess of total inorganic carbon above that present in solution at the specified pH and pCO2 level.   The exchange of CO2 with the atmosphere causes CT to vary.  Because neither electroneutrality nor the proton condition are imposed by the program, alkalinity can not be treated as a conservative quantity, preventing direct solution of certain types of problems.  This limitation can, however, be largely overcome by utilizing the following new features of the program (v. 2.0) that facilitate open-system CO2 calculations:

1.	The charge balance is computed based on the predicted distribution of species, after convergence to a numerical solution.

2.	Similarly, the program computes [Alk] based on the distribution of dissolved carbonate species, and provides this value in the program output.  The computed [Alk] does include contributions from all carbonate complexes and ion pairs such as NaHCO30 and CaCO30, but not from species such as borate.

Table 3.  Summary of some options for performing open-system CO2 computations using v. 2.0 of GEOCHEM-PC.

Type	pH	pCO2	[Alk]	CT	Procedure

1a	unknown	input	known	--	Run as open system with excess CT at several fixed pH 						values.  The pH at which the computed [Alk] = known [Alk], 						or charge balance = 0, is the correct pH.

1b	unknown	input	--	input	Run as open system at several fixed pH values. Find the 						pH at which the atmospheric pool of CO2 ('precipitated' 						H2CO3) is vanishingly small.

2a	input	unknown	known	--	Run as closed system with varying CT until computed [Alk] 						= known [Alk], or charge balance = 0.  Computed pCO2 						is output by program.

2b	input	unknown	--	input	Run as closed system. Computed pCO2 is output.

3	input	input	unknown	unknown	Run as open system with excess CT (CO2 escapes to 						atmsophere).  Computed [Alk] and true CT are output

by program.

3.	The total dissolved carbonate (CT) remaining in solution after equilibration with the atmosphere is calculated and provided in the program output.

4.	When run as a closed system (-log pCO2 set to 0.0), the program computes (Eq. 1) and outputs the theoretical pCO2 in equilibrium with the carbonate species in solution.

These program-computed parameters permit solution of many problems by following the procedures outlined in Table 3, where 'input' parameters are known, either experimentally or for hypothetical cases, and 'unknown' parameters are the objectives of the computation.  For type 1a and 2a problems, [Alk] is 'known', but cannot be directly input into the program.  Note that type 1a, 1b, and 2a problems require that the program be run at several specified pH or pCO2 values, and the correct solution found via interpolation.  Our experience has been that no more than two program runs are required; the first can identify a sufficiently narrow range in pH or pCO2 that the second can locate the correct value exactly.

Please note:  Because of the numerical dichotomies between [Alk] and the TOTH parameter, the computed pH option is generally incompatible with open-system CO2 computations.

For solutions where no solid phases are predicted to precipitate (other than the fictitious H2CO3), either the computed [Alk] or computed charge balance can be used as the criterion for type 1a and 2a problems.  If a solid phase such as calcite is predicted to precipitate, however, alkalinity is effectively lost from the solution phase, and only the computed charge balance of zero is a correct criterion for solving the problem.

Note also that whenever H2CO3 is reported to have dissolved, it has done so completely and there is insufficient inorganic carbon in the system to maintain the specified pCO2 level.  GEOCHEM-PC alerts the user to this loss, and advises that the case be rerun using a higher input concentration of CT.

The input data files EXAMPLE4.DAT and EXAMPLE5.DAT included on the distribution diskette provide examples of some of the computations outlined in Table 3.

Redox Computations

GEOCHEM-PC can compute the distribution of redox species if the redox potential of the system is specified using the master variable pE, but is not designed to predict pE (or Eh) based on input concentrations of redox reactants and products.  The program permits consideration of up to 20 redox reactions in a single program run, and the thermodynamic database may contain up to 30 different redox reactions.  We reiterate the general caution regarding redox computations with geochemical speciation models: inclusion of a redox couple should be based on a reasonable expectation that both reactant and product are present in significant amounts.  Otherwise, the very low concentrations (i.e., < 10-30 mol L-1) for certain species greatly increase the probability of convergence problems and floating-point math errors.

Careful scrutiny of the original GEOCHEM manual is recommended before attempting to use this program feature.  Even so, some users may find the redox nomenclature and protocols perplexing.  Consideration of the redox reactions presently included in the database (Table 4) may help illustrate how GEOCHEM-PC handles redox computations.  Note that the 8-character code names in Table 4 are reproduced in the program output.

Class C reactions (reaction types 10 and -10) are the easiest to grasp.  These involve two metals (e.g., Fe[III] and Fe[II]) or two ligands (e.g., S[VI] and S[-II]), and both members of the redox couple are component species in the thermodynamic database.  By convention, the program assigns the total concentration of metal or ligand to the reactant species, regardless of how the concentration data is entered during input.  Thus, the 'case progress' table (x-A) will indicate zero total concentration for the product metal or ligand, but the free metal/ligand concentrations and activities will be given correctly.  Consultation of the 'primary percentage distribution' table (x-D) will be the easiest way to assess the distribution of the redox species.  For example, both Fe(II) and Fe(III) will be listed, and the sum of the entries under both will be 100% (of the total Fe in the system).

Reaction classes A and B require a more indirect approach.  For these reactions, the product species are not component species, nor combinations of component species.  They are instead treated by the program as a complex or solid phase of the component species appearing on the reactant side of the redox couple.  For Class B reactions (reaction types 1 to 3, negative), a solid phase is the reaction product, and is inserted into the first available 'slot' in the array of three possible solids for the reactants (e.g., Mn[II] and OH- for reaction number 2 in Table 4).  Thus, if there are two 'genuine' Mn(II)-OH solid phases in the database, the predicted quantity of pyrolusite precipitated will be indicated in the program output as 'Mn +2  OH-  Solid #3'.

Class A redox reactions (reaction types 1 through 6, positive) are handled by treating the product species as complexes of the reactant and OH- (for metals), or H+ (for ligands); thus one or both of the tables of complexes (x-B) will be needed.  For the Hg(II) - Hg(I) couple (reaction number 4), where the product is not a component species in the database, the Hg22+ species is given as a 20 0 complex of Hg2+ and OH-.   For the Cr(III) - Cr(VI) couples (numbers 16, 17, and 18), the oxidation involves conversion of a metal to ligand, which GEOCHEM-PC cannot handle directly.  Thus, the Cr(VI) species are given as hydrolysis products of Cr(III) (complexes 4, 5, and/or 6) in the x-B output tables.   Note also that the original Cr(III) hydrolysis reactions 4, 5, and 6 are omitted from the computation if all three redox reactions are considered.  Similarly, even though vanadate is a component species (ligand) in the database, the H2VO4- species must be treated as a 10-4 complex of the VO2+ metal and OH- (reaction number 27).   Note that for ligands such as chromate and vanadate, if the pH range of interest were broad, several different redox reactions would need to specified to account for all possible product species (e.g., H3VO40, H2VO4-, and HVO42-).  Careful judgement should be exercised to determine which reactions are to be included in the computations.

Table 4.  The redox reactions currently present in the thermodynamic database of GEOCHEM-PC.

Reaction			Parameters †

Code

No.	Class	Type ‡	M	L	CM	CL	NENH	Reaction	Name

1	C	-10	6	7	0	0	-1	0	Fe3+ + e-    Fe2+	Fe3/Fe2

2	B	-2	8	99	1	0	2	-4	Mn2+ + 2H2O   MnO2 (pyrolusite) + 4H+ + 2e-	Mn2/MnO2

3	B	-2	14	99	1	0	-2	0	Hg2+ + 2e-    Hg (liq)	Hg2/Hg0

4	A	3	14	99	2	0	-2	0	2Hg2+ + 2e-    Hg22+ (aq)	Hg2/Hg1

5	B	-2	15	99	1	0	2	-4	Pb2+ + 2H2O    PbO2 (s) + 4H+ + 2e-	Pb2/Pb4

6	C	-10	16	17	0	0	1	0	Co2+    Co3+ + e-	Co2/Co3

7	A	3	50	1	0	1	-4	6	CO32- + 6H+ + 4e-    CH2O (aq) + 2H2O	CH2O

8	C	10	2	8	0	0	-8	8	SO42- + 8H+ + 8e-    S2- + 4H2O	SO4/S-2

9	C	10	7	57	0	0	8	-9	NH3 (aq) + 3H2O    NO3- + 9H+ + 8e-	NH3/NO3

10	A	1	50	99	0	0	2	-2	2H2O    H2O2 + 2H+ + 2e-	H2O2

11	C	-10	26	27	0	0	2	0	Sn2+    Sn4+ + 2e-	Sn2/Sn4

12	B	-2	7	8	1	2	2	0	Fe2+ + 2S2-    FeS2 (pyrite) + 2e-	FeS2

13	B	-2	6	99	3	0	-1	-8	3Fe3+ + 4H2O + e-    Fe3O4 (s) + 8H+	Fe3O4(s)

Continued

Table 4. (cont'd)

Reaction			Parameters †

Code

No.	Class	Type ‡	M	L	CM	CL	NE	NH	Reaction	Name

14	B	-3	8	99	3	0	2	-8	3Mn2+ + 4H2O  Mn3O4 (hausmannite) + 8H+ + 2e-	Mn3O4(s)

15	B	-1	8	99	1	0	1	-3	Mn2+ + 2H2O    MnOOH (manganite) + 3H+ + e-	MnOOH

16	A	4	19	99	2	0	6	-14	2Cr3+ + 7H2O    Cr2O72- + 14H+ + 6e-	Cr3/Cr6

17	A	5	19	99	1	0	3	-7	Cr3+ + 4H2O    HCrO4- + 7H+ + 3e-	HCrO4

18	A	6	19	99	1	0	3	-8	Cr3+ + 4H2O    CrO42- + 8H+ + 3e-	CrO4

19	B	-1	50	8	0	1	2	0	S2-    S (s) + 2e-	S-2/S0

20	A	3	50	8	0	4	6	0	4S2-    S42-  + 6e-	4S4-2

21	A	4	50	8	0	5	8	0	5S2-    S52-  + 8e-	5S5-2

22	C	-10	33	9	0	0	1	0	Cu+    Cu2+ + e-	Cu1/Cu2

23	B	-1	33	99	1	0	-1	0	Cu+ + e-    Cu (s)	Cu1/Cu0

24	C	-10	38	37	0	0	1	-4	U4+ + 2H2O    UO2+ + 4H+ + e-	U+4/U+5

25	C	-10	37	32	0	0	1	0	UO2+    UO22+ + e-	U+5/U+6

Continued

Table 4. (cont'd)

Reaction			Parameters †

Code

No.	Class	Type ‡	M	L	CM	CL	NE	NH	Reaction	Name

26	C	-10	39	40	0	0	1	-2	V3+ + H2O    VO+2 + 2H+ + e-	V+3/V+4

27	A	5	40	99	1	0	1	-4	VO2+ + 3H2O    H2VO4- + e- + 4H+ 	V4/H2VO4

28	A	6	40	99	1	0	1	-5	VO2+ + 3H2O    HVO4- + e- + 5H+ 	V4/HVO4

29	B	-1	50	57	0	2	-10	12	2NO3- + 12H+ + 10e-    N2 (g) + 6H2O	N2 (g)

†	M and L are metal and ligand code numbers, CM and CL are their respective stoichiometric coefficients.  NE = the number   of electrons produced, and NH = the number of protons consumed, in the reaction as written.  Note that M and L both refer  to metals (or to ligands) for Type C reactions.

‡	A numerical code used by GEOCHEM-PC to differentiate reaction types.

The input data files EXAMPLE6.DAT and EXAMPLE7.DAT included on the distribution diskette provide examples that illustrate all three classes of redox reaction computations.

Mixed Solids

GEOCHEM-PC can perform computations for 'mixed solids', i.e., solid phases that contain more than one metal, one ligand, and either H or OH.  The program can consider a maximum of 13 mixed solids during a program run, and the database may hold 20 such solids in all.  There are two types: those that contain, at most, two metals, two ligands, plus Al, Si, H, and OH (type 0), and those that can contain up to four metals, plus Al, Si, H, and OH as the only ligand (type 1).  Type 1 reactions are useful for phyllosilicates and similar mineral phases.

The conventions for writing the reactions are virtually identical with those presented in Table 1, and the current selection of mixed solids contained in the database is given in Table 5.  Again, the 8-character code names given appear in the program output.  Please note that virtually all of the mixed solids presently in the database were inherited from GEOCHEM, and we have not checked or evaluated their stability constants.  We suggest that users utilize this feature with caution, and only use it when there are strong indications that the mixed solid of interest is actually present in the experimental system.  Gibbs phase rule violations, lack of convergence, and floating-point math errors are all likely to occur during mixed solids computations.  Disallowing any simple solids that might also precipitate will often increase the probability of successful computation.  If complex solids are a primary research interest, our general advice is to consider other programs better-suited to this task (e.g., MINTEQA2).

The input data file EXAMPLE8.DAT included on the distribution diskette provides an example of a mixed solid computation for the basic aluminum sulfate, alunite.

Adsorption Modeling

We have removed the cation exchange and adsorption models inherited from GEOCHEM, and version 2.0 of GEOCHEM-PC makes no attempt to model these reactions.  Programs such as MINTEQA2 and those from the ...EQL family are specifically designed to model surface adsorption phenomena, and offer a broad selection of adsorption models.

AKNOWLEDGEMENTS

We thank Dr. L. E. Sommers for his assistance with programming changes, and Dr. P. F. Bell for his helpful critiques of the program and contributions to the thermodynamic database.

D. R. Parker 	July 28, 2026

W. A. Norvell

R. L. Chaney

REFERENCES

Davies, C. W. 1962. Ion association. Butterworths, London.

Harned, H. S., and B. B. Owen. 1958. The physical chemistry of electrolytic solutions. Reinhold, New York.

Helgeson, H. C. 1969. Thermodynamics of hydrothermal systems at elevated temperatures and pressures. Am. J. Sci. 267:729-804.

Lindsay, W. L. 1979. Chemical equilibria in soils. John Wiley & Sons, New York.

Martell, A. E., and R. M. Smith. 1976-1989. Critical stability constants. 6 vols. Plenum Press, New York.

Morel, F. M. M., and J. J. Morgan. 1972. A numerical method for computing equilibria in aqueous chemical systems. Environ. Sci. Tech. 6:58-67.

Parker, D. R., L. W. Zelazny, and  T. B. Kinraide. 1987. Improvements to the program GEOCHEM. Soil Sci. Soc. Am. J. 51:488-491.

Sposito, G. 1984. Chemical models of inorganic pollutants in soils. CRC Crit. Rev. Environ. Control. 15:1-24.

Sposito, G. 1986. Corrections to the program GEOCHEM. Soil Sci. Soc. Am. J. 50:270.

Sposito, G., and S. V. Mattigod. 1980. GEOCHEM: A computer program for the calculation of chemical equilibria in soil solution and other natural water systems. Kearney Foundation of Soil Science, Univ. of California, Riverside.

Stumm, W., and J. J. Morgan. 1981. Aquatic chemistry.  2nd ed. John Wiley & Sons, New York.

APPENDIX A - THE MASTEXPL.DAT FILE

LINE 1. Input file for GEOCHEM-PC, v. 2.0.  A descriptive comment follows.

This is a master example data file for GEOCHEM-PC, version 2.0.  It illustrates

LINE 2. Second descriptive comment follows (80 char. max.).

all options.  DO NOT try to run it -- Gibbs phase rule violation will occur.

LINE 3. Maximum number of iterations: usually 25 to 50

50

LINE 4. Convergence criterion: usually 1.0e-4

1.0e-4

LINE 5. Number of metals to be considered, including H+

7

LINE 6. Number of ligands to be considered, including OH-

5

LINE 7. Number of cases to be run (1 to 10)

2

LINE 8. 0=solids can ppt.; -1=none allowed; n=number to be imposed/disallowed

2

LINE 9. pH control: f = fixed pH;  c = pH is calculated by GEOCHEM

c

LINE 10. Redox reactions to be considered?  y = yes  n = no

y

LINE 11. Mixed solids to be considered?  (y or n)

y

LINE 12. Ionic strength to be fixed? y = fixed; n = calculated by GEOCHEM

n

LINE 13. Interaction output: 0=none; 1=capacities; 2=intensities; 3=both

0

LINE 14. Output case progress table (y or n)?

y

LINE 15. Output for complexes?: 0=none; 1=concentrations; 2=activities; 3=both

2

LINE 16. Output overall speciation table (y or n)?

n

LINE 17. Output primary percentage distribution of species table (y or n)?

n

LINE 18. Output table(s) of conditional log K's (y/n)?

n

LINE 19. Fixed or guessed ionic strength (mol/L):

0.010

LINE 20. Metals: code no., -log of free metal guess (1st case), and each case

1 3.1 3.000 3.000

5 2.7 2.678 2.678

6 8.0 4.699 4.699

7 6.3 4.699 4.699

9 8.0 5.000 5.000

33 6.3 5.301 5.301

LINE 21. Ligands: code no., -log of free ligand guess (1st case), and each case

1 4.0 2.301 2.301

3 2.7 2.699 2.699

9 8.0 4.301 4.301

57 2.7 2.699 2.699

LINE 22. pH for each case: measured; or guesses when pH calculated by GEOCHEM-PC

7.000 8.000

LINE 23. TOTH: total H+ (mol/L); used only for calculated pH (neg. value = OH-)

5.00e-4 -5.00e-4

LINE 24. pCO2: -log partial CO2 pressure (0 for closed); used only with CO3

3.500 3.500

LINE 25. pE data: -log electron activity; used only for redox calculations

8.000 8.000

LINE 26. pN2 data: -log partial N2 pressure; used only for redox calcs. w/ NO3-

0.096 0.096

LINE 27. Imposed/disallowed solid phases: see manual (neg. value = disallow)

1 9 1 9 9 -1

LINE 28. Number of redox reactions to consider; used only if item 10 = 'y'

2

LINE 29. Redox reaction code numbers separated by a space. See manual for list.

1 22

LINE 30. Number of mixed solids to consider; used only if item 11 = 'y'

2

LINE 31. Mixed solids code numbers: see manual for list (neg. value = disallow).

10 13

APPENDIX B

COMPONENT SPECIES CURRENTLY IN THE GEOCHEM-PC DATABASE

METALS

Code	Code

No.	Name	Description

01	Ca	Calcium

02 	Mg	Magnesium

03 	Sr	Strontium

04 	K	Potassium

05   	Na	Sodium

06 	Fe +3 	Iron - ferric

07 	Fe +2  	Iron - ferrous

08 	Mn +2  	Manganous

09 	Cu +2 	Copper - cupric

10 	Ba    	Barium

11 	Cd   	Cadmium

12 	Zn   	Zinc

13 	Ni     	Nickelous

14 	Hg +2 	Mercury (II) - mercuric

15 	Pb    	Lead

16 	Co +2	Cobalt - cobaltous

17 	Co +3	Cobalt - cobaltic

18 	Ag   	Silver

19 	Cr +3	Chromic

20 	Al  	Aluminum

21 	Cs  	Cesium

22 	Li  	Lithium

23 	Be  	Berylium

24 	Sc	Scandium

25 	TiO 	Titanium

26 	Sn +2	Tin - stannous

27 	Sn +4	Tin - stannic

28 	La	Lanthanum

29 	Ce	Cerium

30 	Au	Gold

31 	Th	Thorium

32 	UO2 +2	Uranium (VI) - uranyl

33 	Cu +1	Copper - cuprous

34 	CH3Hg	monomethylmercury

35 	Rb  	Rubidium

36 	Me4N 	tetramethylammonium

37 	UO2 + 	Uranium (V)

38 	U +4  	Uranium (IV) - uranous

39 	V +3  	Vanadium (III)

40 	VO +2 	Vandium (IV) - vanadyl

50 	H+    	Hydronium or proton

LIGANDS

01 	CO3  	Carbonate

02 	SO4 	Sulfate

03 	Cl  	Chloride

04 	F   	Fluoride

05 	Br  	Bromide

06 	I   	Iodide

07 	NH3 	Ammonia

08 	S -2 	Sulfide

09 	PO4  	Phosphate

10 	P2O7 	Pyrophosphate

11 	P3O10 	Tripolyphosphate

12 	SiO4  	Silicic acid

13 	S2O3  	Thiosulfate

14 	CN    	Cyanide

15 	OAc   	Acetate

16 	ACAC  	Acetylacetate

17 	Citr 	Citrate

18 	Oxal  	Oxalate

19 	Sali 	Salicylate

20 	Tart 	Tartrate

21 	EN   	Ethylenediamine

22 	DIP 	Dipyridyl

23 	SuSal	Sulfosalicylate

24 	Gly  	Glycine

25 	Glut  	Glutamate

26 	Pico	Picolinate

27 	NTA	Nitrilotriacetate

28 	EDTA	Ethylenediamine-N,N,N',N'-tetraacetate

29 	CDTA	trans-1,2-cyclohexyl-diamine-N,N,N',N'-tetraacetate

30 	CySH	Cysteine

31 	DFOE	Desferri-ferrioxamine-E (Nocardamine)

32 	Phth	Phthalate

33 	Arg  	Arginine

34 	Orn  	Ornithine

35 	Lys 	Lysine

36 	His	Histidine

37 	Asp 	Aspartate

38 	Ser 	Serine

39 	Ala 	Alanine

40 	Tyr 	Tyrosine

41 	Met	Methionine

42 	Val	Valine

43 	Thr 	Threonine

44 	Phe 	Phenylalanine

45 	Iso 	Isoleucine

46 	Leu  	Leucine

47 	Pro  	Proline

48 	B(OH)4	Borate

49 	SO3   	Sulfite

50 	SCN  	Thiocyanate

51 	NH2OH	Hydroxylamine

52 	MoO4 	Molybdate

53 	WO4  	Tungstate

54 	ASO4	Arsenate

55 	HVO4 	Vanadium (V) - vanadate

56 	SeO3	Selenite (Se[IV])

57 	NO3 	Nitrate

58 	DTPA 	Diethylenetriaminepentaacetate

59 	SeO4 	Selenate (Se[VI])

60 	Malea	Maleate

61 	BES 	Benzylsulfonate

62 	ClO4  	Perchlorate

63 	open 	open

64 	open 	open

65 	Form 	Formate

66 	FULV1	Fulvic acid ligand 1 (Sposito & Mattigod)

67 	FULV2 	Fulvic acid ligand 2 (Sposito & Mattigod)

68 	EDDHA 	Ethylene-bis-[2-(o-hydroxyphenyl)glycine] (also EHPG)

69 	HBED	N,N'-bis(2-hydroxybenzyl)-ethylenediamine-N,N'-diacetate

70 	HEDTA	N-2-hydroxyethyl-ethylenediamine-N,N',N'-triacetate

71 	EGTA	Ethylene-bis-(oxyethylenenitrilo)-tetraacetate

72 	Succ	Succinate

73 	Malate	Malate

74 	HEIDA	Hydroxyethyl-imino-diacetic acid

75 	EDDA 	N,N-ethylene-diamine-diacetate (N,N-EDDA)

76 	DFOB 	Desferri-ferrioxamine-B

77 	IDA 	Iminodiacetic acid

78 	FZ  	Ferrozine = PDTS = 3-(3-pyridyl)-5,6-

bis(4-phenylsulfonate)-1,2,4 triazine

79 	oPHEN 	o-phenanthroline = 1,10-phenanthroline

80 	BPDS	Bathophenanthrolinedisulfonate = 4,7-diphenyl-1,10-

phenanthrlolinedisulfonate

81 	MES  	2-(N-morpholino)-ethanesulfonate

82 	Asc 	Ascorbate = Vitamin C

99 	OH-	Hydroxyl

? Parker, D. R., W. A Norvell, and R. L. Chaney. 1995. GEOCHEM-PC: A chemical speciation program for IBM and compatible personal computers. p. 253-269.  In R. H. Loeppert et al. (ed.) Chemical equilibrium and reaction models.  SSSA Spec. Publ. 42, ASA and SSSA, Madison, WI.