Ionic Strength

GEOCHEM-PC can be run with either computed or fixed ionic strength.  The latter option is convenient for complex systems where all of the component species may not be known or experimentally determined (e.g., soil solutions).  In such situations, the solution electrical conductivity can be measured and used to estimate I based on empirical relationships; the estimate is then input to the program as the fixed ionic strength.